SPECIFICATIONS

Materials: T6061 aluminium with hard anodised surface finish
Light source: Cree high intensity LED, chip life 50000hrs
Light output: 6000 Lumen - Colour temperature 6000K - 65000K
Beam angle: 10 degrees spot
Depth rating: 200m/600ft
Swich type: Magnetic 
	Function: 100% - 50% - 15%
Burn time: 70min (100%) - 140min (50%) - 480min (15%)
Net weight: 696g inc battery
Battery type: 3 x 21700 Li-ion rechargeable battery pack.


INSTRUCTIONS AND OPERATIONS

Switch on and off by pressing and holding for 2-3 seconds. To alternate between light levels press for 1 second. If the light is low on power it will flash every 30 seconds.

MAINTENANCE AND STORAGE

Rinse the light in fresh water after every use. Check that the o-rings are clean and not damaged. O-rings should be changed every annually. 
The Li-ion battery should be stored in a clean and dry place. The battery should be charged or discharged every 2-3 weeks.

WARNING

Do not shine the light directly in to eyes.
Do not use the light above water due to the risk of overheating.

Do not open the light housing under water.

WARRANTY

Torch warranty - 1 year
Battery and charger - 6 months

Warranty does not include

Loss or damage to the light

Wear due to regular use ex. Surface scratches
Damage by sun, heat or corrosive chemicals
Overheating due to improper use
Malfunction due to poor maintenance, improper use, alterations or negligence